CardOS API

Version 2.4.1.5

Release Notes

# Contents

Contents	2

1	Introduction	3

1.1	Purpose of the Document	3

1.2	Addressee of the Release	3

1.3	References	3

2	System Requirements (HW/SW)	4

2.1	Hardware	4

2.1.1	Supported Smart Card Products	4

2.1.2	Recommended Smart Card Readers	4

2.2	Software	5

2.2.1	Operating Systems	5

2.2.2	Applications	5

3	Object of the release	6

3.1	Released components	6

4	Changes compared to previous Release	7

4.1	New features	7

4.2	Fixed Bugs	7

4.3	Implemented / closed CR	7

Description	7

4.4	Known Errors/Limitations of this Version	7

4.5	Warnings	8

4.5.1	Product Installation	8

5	Log File Management of CardOS API Sub-Modules	9

6	CR handling and reporting	10

6.1	How to report a CR	10

Description	10

6.2	Error report addressee	11

# Introduction

## Purpose of the Document

The purpose of this “CardOS API Version 2.4.1 – Release Notes” document is to provide a list of all the characteristics of the new CardOS API Version 2.4.1 (referred to as CardOS API).

## Addressee of the Release

This document is intended to be a reference for both end users and developers who use applications that can be integrated with personalized CSP and standard PKCS#11.

## References

Further reference documents for this release are:

- [R4]	CardOS API Version 2.4.1 - Modules Manual

# System Requirements (HW/SW)

This section lists the hardware and software configurations compliant with the CardOS API. Other configurations not yet tested may be added to succeeding versions of the CardOS API.

## Hardware

The module has been successfully installed and tested on personal computers equipped with Intel processors (486 and Pentium) and the following smart cards and smart card readers:

### Supported Smart Card Products

The smart card products supported are:

  - CardOS/M4.00

  - CardOS/M4.01

  - CardOS/M4.01a

  - CardOS/M4.10

  - CardOS/M4.20

  - CardOS/M4.20 (64 kb)

The USB token supported is:

- Eutron CryptoIdentity ITSEC

### Recommended Smart Card Readers

CardOS API has been successfully tested with the following PC/SC readers:

| List of recommended PC/SC readers Manufacturer | Model | Connection type | Driver type |
| --- | --- | --- | --- |
| ActivCard | ACTR-01 | Serial | PC/SC |
| Eutron | CrytoIdentity | USB Token | PC/SC |
| Gemplus | GemPC 410 | Serial | PC/SC |
| Gemplus | GCR 410 | Serial | PC/SC |
| Gemplus | GemPC 430 | USB | PC/SC |
| IPM-Net | MiniLector | USB | PC/SC |
| IPM-Net | MiniLector | Serial | PC/SC |
| Towitoko | Chip Drive Micro100 | Serial | PC/SC |
| Omnikey | Card Man 1010 | Serial | PC/SC |
| Omnikey | Card Man 2010 | Serial | PC/SC |
| Omnikey | Card Man 2020 | USB | PC/SC |
| Omnikey | Card Man 3121 | USB | PC/SC |
| Omnikey | CardManMobile | PCMCIA | PC/SC |
| Precise Biometrics | PB 100 MC | USB | PC/SC |

Before the installation, be sure the smart card reader is installed correctly on your system. Refer to the reader manual from your preferred manufacturer.

(For some Operating Systems the installation of CardOS API may fail if no reader is correctly installed)

Warning: Any PC/SC compliant smart card reader should work fine. However, occasionally problems may arise for various reasons (for example some smart card based operations are not compatible with the timeout of the reader, etc.). Readers contained in the above list are recommended.

## Software

### Operating Systems

The CardOS API has been tested on the following Operating Systems:

- MS Windows 95 OSR2

- MS Windows 98

- MS Windows 98 SE

- MS Windows NT 4.0 (Service Pack 6 or higher)

- MS Windows 2000

- MS Windows XP

- MS Windows 2003

Please note that MS Windows 2000, XP and 2003 operating systems have many native features that extend basic and security functionalities by the use of smart cards. This CardOS API implementation allows the user to integrate all these functionalities.

To get a full overview of MS operating systems’ smart card based functionalities please refer to the Microsoft documentation.

### Applications

As CardOS API consists of a CardOS_PKCS11 module and a CardOS_CSP module applications using one or both of these interfaces can be supported. As sometimes such applications do not comply with these interfaces entirely only the applications tested with CardOS API are recommended. Nevertheless it is possible that other applications run with the CardOS API correctly.

The following tables show all the third party products that were successfully tested with CardOS API.

| Applications | Version | Use |
| --- | --- | --- |
| Netscape Navigator | 4.72, 4.73, 4.76, 4.77, 4.78, 4.79, 6.2, 7.0 | SSL v3 strong user authentication to establish an https connection with client and server authentication |
| Netscape Messenger | 4.72, 4.73, 4.76, 4.77, 4.78, 4.79, 7.0 | Signature and encryption of mail messages |
| Mozilla | 0.9.8, 1.1, 1.2 | Web browser and mail client |
| Internet Explorer | 5.0, 5.5, 6.0 | SSL v3 strong user authentication to establish an https connection with client and server authentication |
| Microsoft Outlook Express | 5.0, 5.5, 6.0 | Signature and encryption of mail messages |
| Microsoft Outlook | 2000, 2002 | Signature and encryption of mail messages |

| CA Products | Version |
| --- | --- |
| Microsoft Certification Authority | 4.0, 5.0, 6.0 |

# Object of the release

## Released components

| Component name | Related File | Brief description |
| --- | --- | --- |
| SI_PKCS11.dll | <system dir>\ SI_PKCS11.dll | Main PKCS#11 module |
| SI_Csp.dll | <system dir>\SI_Csp.dll | Main CSP module |
| SICore.dll | <system dir>\ SICore.dll | High Level interface module |
| SiModule.dll | <system dir>\ SiModule.dll | Utility module |
| SISCCom.dll | <system dir>\ SISCCom.dll | PC/SC interface module |
| SICrypto.dll | <system dir>\ SICrypto.dll | Cryptographic interface module |
| SISCMon.exe | <system dir>\SISCMon.exe | Certificate manager utility |
| SiMngr.dll | <system dir>\ SiMngr.dll | Utility module |
| SiAsn1.dll | <system dir>\ SiAsn1.dll | Ans1 parser module |
| SiCertifViewer.dll | <system dir>\ SiCertifViewer.dll | Utility module |
| SiCrtviewer.dll | <system dir>\ SiCrtviewer.dll | Utility module |

# Changes compared to previous Release

## New features

The main change of this version of CardOS API has been the modification of the software architecture. The old implementation was based on an out-of-process COM module, the new one is based on an in-process DLL which enables CardOS API to work in Terminal Server environments.

All components of CardOS API have been changed. This release has added many new features without losing compatibility with previous releases and previous smart cards.

Here is a list of the main features implemented.

- Terminal Server support: CardOS API can be used in Terminal service environments. On Windows XP it is possible to use the Remote Desktop with smart cards.

- Dynamic ATR: The CSP interface of CardOS API has been upgraded to manage any ATR for CardOS smart cards.

- Dynamic Object creation: In the previous version of CardOS API it was only possible to create a fixed number of RSA/Certificate objects on the smart card (the number was specified by the personalizers). Using this new release (in conjunction with the new GDO version 2) it is possible to dynamically create any object.

- Certificate insertion/removal: To enrich the functionality of the CSP interface, CardOS API has added a tool that manages automatically the certificates present on CardOS smart cards. This tool adds the certificates to Microsoft Store when a smart card is inserted in a smart card reader present on the system. The certificates are removed when the smart card is extracted.

- New PKCS11 functions/mechanism: Please refer to [R4] to have a complete overview about the new functions/mechanisms added to CardOS API.

- Biometric Match on Card: The mechanism of Match on Card of CardOS smart card has been implemented on CardOS API. This mechanism enables CardOS smart card to used fingerprint verification mechanism instead of traditional PIN.

## Fixed Bugs

These are fixed bugs:

| CR No. | Description |
| --- | --- |
| 1 | Solved all problems about instability of logon functionality. |
| 2 | Solved problems about smart card service thread |
| 3 | Solved Problem about a creation of data object and generation RSA keys. |
| 4 | Solved Problem on c_CreateObject for read only token objects |
| 5 | Solved Problem on AC setting on private objects |
| 6 | Solved Problem on import public key RSA |

## Implemented / closed CR

| CR No. | #### Description |
| --- | --- |
| 1 | CSP specification “Certificate last in first out” it’s used always the last certificate inserted. |
| 2 | Remote Desktop connection between Windows XP/2000 clients and Windows 2003 Server and between two Windows XP clients. |
| 3 | Pin cache, the PIN’s request is made ones for csp session. |
| 4 | It is now possibile to call c_WaitForSlotEvent without c_GetSlotList. It is possible to catch the reader event without loading the reader list. |

## Known Errors/Limitations of this Version

- With the operating systems Windows 95, Windows 98 and Windows NT, it is not possible to use more than one smart card reader containing a CardOS M4.0x smart card with the CardOS API.

- The Microsoft native Winlogon process fails if SI_CSP is not the first entry in the registry key: 
HKEY_LOCAL_MACHINE\Software\Microsoft\Cryptography\Calais\SmartCards 
If you need to use the Win Logon, please ensure that the first found entry in the above registry key is that of CardOS API (Eventually the original registry key of the CardOS API must be copied to a key with a name which is the first in the entries list.).

- The credential of a user created by Entrust Entelligence cannot be propagated on the system store using Scmon or CardIntro. This limitation doesn’t allow the usage of Entrust certificates by CardOS_CSP. It only works with the Entrust Desktop Solutions.

- If RSA key pairs are generated with software mechanism (i.e. key generation outside the smart card in software) the browser Netscape crashes. So always use the On-Chip-Key-Generation (default setting) with the Netscape browser.

- If more than one smart card is inserted before starting VPN and/or RAS (functionalities of the operating systems Windows XP and Windows 2000) connection, the operation fails.

- If two smart card are inserted or removed contemporarily not all the certificates will be propagated.

- Before using a smart card wait until the propagation of its certificates is ended.

- Before logging out wait until the propagation of its certificates is ended.

- In case it is necessary to Plug/Unplug smart card readers, it is necessary to restart the workstation or to close the user session.

- CardOS API slows down until the certificate propagation is in progress.

- Remote Desktop connection between two Windows XP clients hangs the automatic propagation of certificates on the host workstation.

## Warnings

### Product Installation

Before installing CardOS API, be sure to have installed a smart card reader. Otherwise the installation may not terminate correctly.

To install CardOS API it is mandatory to have admin rights of the workstation.

# Log File Management of CardOS API Sub-Modules

This release of the CardOS API allows the user to log some run-time information about the behaviour of the library. It’s also possible to set the level of detail of such information.

Due to the fact that such functionality is hidden to the host application, enabling the production of a log file means adding a pair of particular keys to the Windows registry; such keys will cause the application to produce the desired file.

In detail the DWORD keys named

- “SI_PKCS11” for the SI_PKCS11 and

- “SICSP” for the SI_CSP

should be added to the registry folder “HKEY_LOCAL_MACHINE\ Software\CARDOS_API_241”.

The following log levels are available:

- Log level 0 means no log at all

- Log level 1 reports names, arguments and return values for CardOS modules API.

- Log level 2 reports run time information about accessory internal functions.

- Log level 3 reports information about very low-level functions (such as memory management).

All log information (for both SI_PKCS11 and SI_CSP) will be stored in 2 files called

  - “PKCS11.log” and

  - “SICSP.log”

The files are created in the “C:\” drive of your PC.

Remove the registry keys when log information is not required. Log writing is a heavy operation that could cause loss of performance in the host application.

By default, the driver doesn’t print any log file.

For a complete documentation about CardOS API modules, refer to [R4].

The log is written by a circular log manager. Every time an event is logged, the log manager appends a log to the log file. Every 50 logged events the log manager checks the length of the log file, and cuts the beginning lines in order to make the log file 1000 lines long. So, if the logged event produces one line on the log file, the log file will be 1000 to 1050 lines long. The length of the log and the periodicity of the length check are hardcoded, and the mazimum size of the single log line is 10000 characters.

# CR handling and reporting

Every time a new problem is detected on a build of the product, it can be reported to the CardOS development team. Consider that CRs may arise from both host applications and CardOS modules. Actually only CRs concerning CardOS modules will be taken in consideration. For this reason, end users who want to report a CR may skip this section and directly refer to the distribution point which is able to report the problem to the CardOS development team in a correct way.

## How to report a CR

Every time a new problem will be identified, you can submit a CR to the CardOS development team following the template below.

| Fields (fields in bold are mandatory) | Description | Example |
| --- | --- | --- |
| Name | The name of the person (or distribution point) who is submitting the CR | Mario Rossi |
| Role | The role in using CardOS modules, played by the person who is submitting the CR (E.g. developer, end user….) | Developer |
| Company | Specify the company name (or the particular department) in which is working the person who is submitting the CR | Siemens Informatica |
| Email Address | Email address for receiving answers from the CardOS development team | mario.rossi@siemens.com |
| Date | The date when problem occurred | 15/12/2001 |
| Product version used | CardOS API version used | 2.x.x |
| Operating System | The operating system used | Windows 2000 professional |
| Smart Card Reader | Smart card reader used. | Omnikey 1010 serial interface |
| Host application | Host application which is using the services of CardOS modules when the problem occurred | Outlook Express |
| #### Description | Description of the problem. Describe in detail the situation in which the problem appeared and all its effects. | While sending email messages, the software is unable to sign with the Smart Card |
| Log files | Refer to section 5.2 | See PKCS11_log file attached |
| File System | Insert here all the technical information regarding the smart card File System, in use when the problem occurred. If possible, attach to email the packages used to build the card File System. | –For example: See the attached script, which is useful to generate a smart card file system like the one in which the problem occurred. |

## Error report addressee

To report an error, send an email, including all the information indicated in the template above and all the possible attachments, to your reseller.